# ALL COMMANDS (ONE PAGE — COPY & PASTE READY)

show run                     // Show current config

! Console Password

line console 0               // Select console

password abc@123            // Set console password

login                       // Enable password checking

exit

! AUX Password

line aux 0                   // Select AUX line

password abc@123            // Set AUX password

login

! Enable Passwords

enable password abcd@R1      // Normal enable password

enable secret abcd@R1        // Encrypted enable password

service password-encryption  // Encrypt all passwords

security passwords min-length 8    // Minimum password length

security password lifetime 30      // Password expiry days

security passwords history 5       // Prevent reuse of 5 old passwords

wr                          // Save config

###########################

#  PASSWORD RECOVERY

###########################

(1) Power cycle router

(2) Press Ctrl + Break to enter ROMMON

rommon 1> confreg 0x2142     // Ignore startup-config on boot

rommon 2> reset              // Reboot router

R1# copy startup-config running-config   // Restore config

R1(config)# enable secret NEWpassword123 // Set new enable password

R1(config)# config-register 0x2102       // Return to normal boot

R1# wr                                   // Save config

# ALL COMMANDS (ONE PAGE) — TELNET + SSH

##############################

# IP CONFIGURATION

##############################

R1:

int fa0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

int fa0/1

ip address 192.168.1.3 255.255.255.0

no shutdown

R2:

int fa0/0

ip address 192.168.1.5 255.255.255.0

no shutdown

int fa0/1

ip address 192.168.1.4 255.255.255.0

no shutdown

##############################

# TELNET CONFIGURATION (R1)

##############################

username Test privilege 15 secret Test@123

line vty 0 4

login local

transport input telnet

exit

wr

# Test from R2:

telnet 192.168.1.1

##############################

# SSH CONFIGURATION (R1)

##############################

hostname Router1

ip domain-name test.lab

username Admin privilege 15 secret Admin@R2

crypto key generate rsa

(1024)

line vty 0 4

login local

transport input ssh

exit

show ip ssh

# Test from R2:

ssh -l Admin 192.168.1.1

# ALL COMMANDS — ONE PAGE (COPY–PASTE READY)

############################################

# EIGRP CONFIGURATION (R1)

############################################

router eigrp 100

network 192.168.12.0 0.0.0.255

network 192.168.1.0 0.0.0.255

no auto-summary

exit

key chain EIGRP_KEYS

key 1

key-string eigrp123

exit

interface s1/0

ip authentication mode eigrp 100 md5

ip authentication key-chain eigrp 100 EIGRP_KEYS

exit

############################################

# EIGRP CONFIGURATION (R2)

############################################

router eigrp 100

network 192.168.12.0 0.0.0.255

network 192.168.2.0 0.0.0.255

exit

key chain EIGRP_KEYS

key 1

key-string eigrp123

exit

interface s1/0

ip authentication mode eigrp 100 md5

ip authentication key-chain eigrp 100 EIGRP_KEYS

exit

############################################

# OSPF CONFIGURATION (R2)

############################################

router ospf 1

network 192.168.23.0 0.0.0.255 area 0

network 192.168.2.0 0.0.0.255 area 0

exit

interface s1/1

ip ospf message-digest-key 1 md5 ospf123

ip ospf authentication message-digest

exit

############################################

# OSPF CONFIGURATION (R3)

############################################

router ospf 1

network 192.168.23.0 0.0.0.255 area 0

network 192.168.3.0 0.0.0.255 area 0

exit

interface s1/1

ip ospf message-digest-key 1 md5 ospf123

ip ospf authentication message-digest

no shutdown

exit

############################################

# REDISTRIBUTION (R2)

############################################

router ospf 1

redistribute eigrp 100 subnets

exit

router eigrp 100

redistribute ospf 1 metric 10000 100 255 1 1500

exit

# ALL COMMANDS — ONE PAGE (Copy–Paste Ready)

#########################################################

# R1 RIP v2 + Authentication

#########################################################

router rip

version 2

no auto-summary

network 192.168.1.0

network 192.168.12.0

exit

key chain RIP

key 1

key-string mypass

exit

interface s1/0

ip rip authentication mode text

ip rip authentication key-chain RIP

exit

#########################################################

# R2 RIP v2 + Authentication

#########################################################

router rip

version 2

no auto-summary

network 192.168.12.0

network 192.168.23.0

exit

key chain RIP

key 1

key-string mypass

exit

interface s1/0

ip rip authentication mode text

ip rip authentication key-chain RIP

exit

interface s1/1

ip rip authentication mode text

ip rip authentication key-chain RIP

exit

#########################################################

# R3 RIP v2 + Authentication

#########################################################

router rip

version 2

no auto-summary

network 192.168.23.0

network 192.168.3.0

exit

key chain RIP

key 1

key-string mypass

exit

interface s1/0

ip rip authentication mode text

ip rip authentication key-chain RIP

exit

# ULL ONE-PAGE COMMAND LIST (EXAM READY)

#############################

# R1 OSPF

#############################

hostname R1

router ospf 1

network 10.0.0.0 0.0.0.255 area 0

network 20.0.0.0 0.0.0.3 area 0

exit

#############################

# R1 AAA + TACACS

#############################

enable password abc

username test password 12345

aaa new-model

tacacs-server host 10.0.0.100

tacacs-server key ccna

aaa authentication login default group tacacs+ local

line vty 0 4

login authentication default

exit

#############################

# R2 OSPF

#############################

hostname R2

router ospf 1

network 30.0.0.0 0.0.0.255 area 0

network 20.0.0.0 0.0.0.3 area 0

exit

#############################

# R2 AAA + TACACS

#############################

enable password xyz

username test1 password 12345

aaa new-model

tacacs-server host 10.0.0.100

tacacs-server key ccna

aaa authentication login default group tacacs+ local

line vty 0 4

login authentication default

exit

# ALL COMMANDS — EXAM ONE-PAGE SUMMARY

###############################

# R1 Static Route

###############################

ip route 0.0.0.0 0.0.0.0 10.0.0.2

###############################

# R2 Static Routes

###############################

ip route 192.168.1.0 255.255.255.0 10.0.0.1

ip route 192.168.2.0 255.255.255.0 10.0.0.6

###############################

# STANDARD ACL (R2)

###############################

access-list 10 deny 192.168.1.10

access-list 10 permit any

interface s2/1

ip access-group 10 in

exit

###############################

# EXTENDED ACL (R2)

###############################

access-list 100 permit icmp 192.168.3.0 0.0.0.255 192.168.2.0 0.0.0.255

access-list 100 deny ip any any

interface s2/1

ip access-group 100 in

exit

###############################

# NAMED EXTENDED ACL (R2)

###############################

ip access-list extended HTTPS_ONLY

permit tcp 192.168.3.0 0.0.0.255 192.168.2.0 0.0.0.255 eq 443

deny ip any any

exit

interface s2/0

ip access-group HTTPS_ONLY in

exit

# PART 4 — PAT (PORT ADDRESS TRANSLATION)

Goal:
Use interface fa0/0 public IP (200.1.1.2) for MANY LAN hosts.

### Step 1 — Define NAT Pool again (if required)

R1(config)# ip nat pool NAT_POOL 200.1.1.2 200.1.1.10 netmask 255.255.255.248

### Step 2 — ACL to permit inside network

R1(config)# access-list 1 permit 192.168.10.0 0.0.0.255

### Step 3 — PAT Command

R1(config)# ip nat inside source list 1 interface fa0/0 overload

➡️ Allows many users to share one public IP.

# ⭐ ONE-PAGE EXAM COMMAND SHEET

###################################################

# R1 Interfaces

###################################################

int fa0/0

ip address 200.1.1.2 255.255.255.248

no shut

exit

int fa0/1

ip address 192.168.10.1 255.255.255.0

no shut

exit

router ospf 1

network 200.1.1.0 0.0.0.3 area 0

network 192.168.10.0 0.0.0.255 area 0

exit

###################################################

# R2 Interfaces

###################################################

int fa0/0

ip address 200.1.1.1 255.255.255.248

no shut

exit

int fa0/1

ip address 100.1.1.1 255.255.255.252

no shut

exit

router ospf 1

network 200.1.1.0 0.0.0.3 area 0

network 100.1.1.0 0.0.0.3 area 0

exit

###################################################

# STATIC NAT

###################################################

ip nat inside source static 192.168.10.10 200.1.1.2

int fa0/1

ip nat inside

exit

int fa0/0

ip nat outside

exit

###################################################

# DYNAMIC NAT

###################################################

no ip nat inside source static 192.168.10.10 200.1.1.2

ip nat pool NAT_POOL 200.1.1.2 200.1.1.10 netmask 255.255.255.248

access-list 1 permit 192.168.10.0 0.0.0.255

ip nat inside source list 1 pool NAT_POOL

###################################################

# PAT (NAT OVERLOAD)

###################################################

ip nat pool NAT_POOL 200.1.1.2 200.1.1.10 netmask 255.255.255.248

access-list 1 permit 192.168.10.0 0.0.0.255

ip nat inside source list 1 interface fa0/0 overload

# TOPIC 7 — CONFIGURING PBR (SOURCE-ONLY + DESTINATION-ONLY)

# 🖼 TOPOLOGY (Based on your notes)

### SOURCE-BASED PBR

200.0.1.1/30             200.0.1.2/30

R1 --------------------- R3

| \                      /

|  \200.0.2.1/30   200.0.2.2/30

|   \

|    R2

|   192.168.0.1/24

|

Loopbacks on R3:

10.0.0.0/29 – L0

10.0.1.0/24 – L1

10.0.2.0/24 – L2

10.0.3.0/29 – L3

# ⭐ PART–1: CONFIGURE EIGRP IN ALL ROUTERS

### R1

R1(config)# router eigrp 10

R1(config-router)# network 200.0.1.0 0.0.0.3

R1(config-router)# network 200.0.2.0 0.0.0.3

R1(config-router)# exit

### R2

R2(config)# router eigrp 10

R2(config-router)# network 100.0.1.0 0.0.0.3

R2(config-router)# exit

### R3

R3(config)# router eigrp 10

R3(config-router)# network 200.0.1.0 0.0.0.3

R3(config-router)# network 200.0.2.0 0.0.0.3

R3(config-router)# network 10.0.0.0 0.0.255.255

R3(config-router)# exit

# ⭐ PART–2: CONFIGURE LOOPBACKS ON R3

R3(config)# interface loopback 0

R3(config-if)# ip address 10.0.0.1 255.255.255.248

R3(config-if)# exit

R3(config)# interface loopback 1

R3(config-if)# ip address 10.0.1.1 255.255.255.0

exit

R3(config)# interface loopback 2

R3(config-if)# ip address 10.0.2.1 255.255.255.0

exit

R3(config)# interface loopback 3

R3(config-if)# ip address 10.0.3.1 255.255.255.248

exit

# ⭐ PART–3: PBR SOURCE-BASED CONFIG (On R1)

Goal:
Send traffic from 10.0.0.0/24 & 10.0.1.0/24 via interface Serial1/0
and
Send 10.0.2.0/24 & 10.0.3.0/24 via next-hop 200.0.2.2

## 🔵 Step–1: Create Source ACLs

### ACL 10 → For Lo0 + Lo1

R1(config)# access-list 10 permit 10.0.0.0 0.0.0.255

R1(config)# access-list 10 permit 10.0.1.0 0.0.0.255

➡️ Matches source IPs Lo0 and Lo1.

### ACL 20 → For Lo2 + Lo3

R1(config)# access-list 20 permit 10.0.2.0 0.0.0.255

R1(config)# access-list 20 permit 10.0.3.0 0.0.0.255

➡️ Matches source IPs Lo2 and Lo3.

## 🔵 Step–2: Create Route-Map for PBR (Source Based)

### Route-map entry 5 → For ACL 10

R1(config)# route-map PBR_1 permit 5

R1(config-route-map)# match ip address 10

R1(config-route-map)# set interface serial 1/0

R1(config-route-map)# exit

➡️ Traffic from Lo0/Lo1 goes via interface s1/0.

### Route-map entry 10 → For ACL 20

R1(config)# route-map PBR_1 permit 10

R1(config-route-map)# match ip address 20

R1(config-route-map)# set ip next-hop 200.0.2.2

R1(config-route-map)# exit

➡️ Traffic from Lo2/Lo3 goes via next hop 200.0.2.2.

## 🔵 Step–3: Apply PBR on LAN interface

R1(config)# interface fa0/0

R1(config-if)# ip policy route-map PBR_1

R1(config-if)# exit

➡️ Tells R1 to apply PBR to incoming packets.

# ⭐ SOURCE PBR COMPLETED ✔

# ⭐ PART–4: DESTINATION-BASED PBR (On R1)

Goal:

- If destination 10.0.0.0/24 → send to 172.16.0.0/24 via 200.0.1.2

- If destination 10.0.2.0/24 → send via 200.0.2.2

## 🔵 Step–1: Set Bandwidth (as in your notes)

R1(config)# interface serial 1/0

R1(config-if)# bandwidth 1024

R1(config-if)# exit

➡️ Sets bandwidth for PBR decision.

## 🔵 Step–2: Create Destination ACLs

### ACL Route-1 (to send via next-hop 200.0.1.2)

R1(config)# ip access-list extended ROUTE_1

R1(config-ext-nacl)# permit ip 10.0.0.0 0.0.0.255 172.16.0.0 0.0.0.255

R1(config-ext-nacl)# exit

### ACL Route-2 (to send via next-hop 200.0.2.2)

R1(config)# ip access-list extended ROUTE_2

R1(config-ext-nacl)# permit ip 10.0.2.0 0.0.0.255 50.0.0.0 0.0.0.255

R1(config-ext-nacl)# exit

## 🔵 Step–3: Create Route-Map for Destination PBR

### Entry 5 → match ROUTE_1

R1(config)# route-map D_PBR permit 5

R1(config-route-map)# match ip address ROUTE_1

R1(config-route-map)# set ip next-hop 200.0.1.2

R1(config-route-map)# exit

### Entry 10 → match ROUTE_2

R1(config)# route-map D_PBR permit 10

R1(config-route-map)# match ip address ROUTE_2

R1(config-route-map)# set ip next-hop 200.0.2.2

R1(config-route-map)# exit

## 🔵 Step–4: Apply PBR on Interface

R1(config)# interface s1/2

R1(config-if)# ip policy route-map D_PBR

R1(config-if)# exit

# ⭐ ONE-PAGE EXAM SHEET (COPY THIS DIRECTLY)

##############################################

# SOURCE BASED PBR

##############################################

access-list 10 permit 10.0.0.0 0.0.0.255

access-list 10 permit 10.0.1.0 0.0.0.255

access-list 20 permit 10.0.2.0 0.0.0.255

access-list 20 permit 10.0.3.0 0.0.0.255

route-map PBR_1 permit 5

match ip address 10

set interface serial 1/0

exit

route-map PBR_1 permit 10

match ip address 20

set ip next-hop 200.0.2.2

exit

interface fa0/0

ip policy route-map PBR_1

exit

##############################################

# DESTINATION BASED PBR

##############################################

interface serial 1/0

bandwidth 1024

exit

ip access-list extended ROUTE_1

permit ip 10.0.0.0 0.0.0.255 172.16.0.0 0.0.0.255

exit

ip access-list extended ROUTE_2

permit ip 10.0.2.0 0.0.0.255 50.0.0.0 0.0.0.255

exit

route-map D_PBR permit 5

match ip address ROUTE_1

set ip next-hop 200.0.1.2

exit

route-map D_PBR permit 10

match ip address ROUTE_2

set ip next-hop 200.0.2.2

exit

interface s1/2

ip policy route-map D_PBR

exit

# SITE-TO-SITE VPN (IPsec Tunnel Mode)

# 🖼 TOPOLOGY (Based on your notes)

LAN1                         WAN                        LAN2

192.168.10.0/24    20.20.20.0/30     30.30.30.0/30     192.168.20.0/24

SW ---- R2 ---------------- R1 ---------------- R3 ---- SW

OSPF AREA 0

- R2 LAN: 192.168.10.0/24

- R3 LAN: 192.168.20.0/24

- Tunnel endpoints:

  - R2 public IP = 20.20.20.1

  - R3 public IP = 30.30.30.2

# ⭐ STEP 1 — Phase 1 (ISAKMP Policy) — on R2 & R3

### 🔵 R2:

R2(config)# crypto isakmp policy 10

R2(config-isakmp)# encryption aes 256

R2(config-isakmp)# hash sha256

R2(config-isakmp)# authentication pre-share

R2(config-isakmp)# group 14

R2(config-isakmp)# lifetime 3600

R2(config-isakmp)# exit

➡️ Defines Phase-1 security parameters.

### Pre-Shared Key:

R2(config)# crypto isakmp key cisco123 address 30.30.30.2

➡️ Key must match on both routers.

# ⭐ STEP 2 — Phase 2 Transform Set (IPsec)

### 🔵 R2:

R2(config)# crypto ipsec transform-set TRANSFORM esp-aes esp-sha-hmac

R2(cfg-crypto-trans)# mode tunnel

R2(cfg-crypto-trans)# exit

➡️ Defines encryption/auth in IPsec tunnel.

# ⭐ STEP 3 — ACL for “Interesting Traffic”

### 🔵 R2:

R2(config)# access-list 100 permit ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255

➡️ Traffic that should enter the VPN tunnel.

# ⭐ STEP 4 — Crypto Map Configuration

### 🔵 R2:

R2(config)# crypto map VPN-MAP 10 ipsec-isakmp

R2(config-crypto-map)# set peer 30.30.30.2

R2(config-crypto-map)# set transform-set TRANSFORM

R2(config-crypto-map)# match address 100

R2(config-crypto-map)# exit

➡️ Binds peer, transform-set, and interesting traffic.

# ⭐ STEP 5 — Apply Crypto Map on WAN Interface

R2(config)# interface serial1/0

R2(config-if)# crypto map VPN-MAP

R2(config-if)# exit

➡️ Activates VPN on outgoing interface.

# ⭐ R3 CONFIGURATION (Mirror of R2)

### Phase-1 ISAKMP:

R3(config)# crypto isakmp policy 10

R3(config-isakmp)# encryption aes 256

R3(config-isakmp)# hash sha256

R3(config-isakmp)# authentication pre-share

R3(config-isakmp)# group 14

R3(config-isakmp)# lifetime 3600

R3(config-isakmp)# exit

### Matching Key:

R3(config)# crypto isakmp key cisco123 address 20.20.20.1

➡️ Matches R2's configuration.

### Phase-2 Transform:

R3(config)# crypto ipsec transform-set TRANSFORM esp-aes esp-sha-hmac

R3(cfg-crypto-trans)# mode tunnel

R3(cfg-crypto-trans)# exit

### ACL for interesting traffic:

R3(config)# access-list 100 permit ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

### Crypto map:

R3(config)# crypto map VPN-MAP 10 ipsec-isakmp

R3(config-crypto-map)# set peer 20.20.20.1

R3(config-crypto-map)# set transform-set TRANSFORM

R3(config-crypto-map)# match address 100

R3(config-crypto-map)# exit

### Apply on interface:

R3(config)# interface serial1/0

R3(config-if)# crypto map VPN-MAP

R3(config-if)# exit

# ⭐ VERIFYING VPN

### Check ISAKMP (Phase 1):

show crypto isakmp sa

### Check IPsec (Phase 2):

show crypto ipsec sa

✔ You should see MM_ACTIVE, QM_IDLE, and incrementing packet counters.

# ⭐ ONE-PAGE EXAM COMMAND SHEET

###########################################

# R2 CONFIG

###########################################

crypto isakmp policy 10

encryption aes 256

hash sha256

authentication pre-share

group 14

lifetime 3600

exit

crypto isakmp key cisco123 address 30.30.30.2

crypto ipsec transform-set TRANSFORM esp-aes esp-sha-hmac

mode tunnel

exit

access-list 100 permit ip 192.168.10.0 0.0.0.255 192.168.20.0 0.0.0.255

crypto map VPN-MAP 10 ipsec-isakmp

set peer 30.30.30.2

set transform-set TRANSFORM

match address 100

exit

interface serial1/0

crypto map VPN-MAP

exit

###########################################

# R3 CONFIG

###########################################

crypto isakmp policy 10

encryption aes 256

hash sha256

authentication pre-share

group 14

lifetime 3600

exit

crypto isakmp key cisco123 address 20.20.20.1

crypto ipsec transform-set TRANSFORM esp-aes esp-sha-hmac

mode tunnel

exit

access-list 100 permit ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

crypto map VPN-MAP 10 ipsec-isakmp

set peer 20.20.20.1

set transform-set TRANSFORM

match address 100

exit

interface serial1/0

crypto map VPN-MAP

exit

###########################################

# VERIFICATION

###########################################

show crypto isakmp sa

show crypto ipsec sa

# OPIC 9 — IPsec + DMVPN CONFIGURATION (HUB-SPOKE)

# 🖼 TOPOLOGY (from your diagram)

ISP CLOUD (Public)

----------------------------------------------------

R1(HUB) Tunnel0: 172.16.0.1/24

|              \

10.1.1.0/29           \

|               \

LAN               \

R2 (Spoke1)

Tunnel0: 172.16.0.2/24

LAN 192.168.10.0/24

R3 (Spoke2)

Tunnel0: 172.16.0.3/24

LAN 192.168.20.0/24

Public IPs:

- R1 public: 10.1.1.1

- R2 public: 192.168.10.1

- R3 public: 192.168.20.1

Tunnel IPs (DMVPN):

- R1: 172.16.0.1

- R2: 172.16.0.2

- R3: 172.16.0.3

# ⭐ STEP 1 — HUB CONFIGURATION (R1)

### 🔵 Create Tunnel Interface

R1(config)# interface tunnel0

R1(config-if)# ip address 172.16.0.1 255.255.255.0

R1(config-if)# no ip redirects

R1(config-if)# tunnel source f1/1

R1(config-if)# tunnel mode gre multipoint

R1(config-if)# ip nhrp network-id 10

R1(config-if)# ip nhrp map multicast dynamic

R1(config-if)# exit

Purpose:

- mGRE: allows multiple spokes

- NHRP: allows hubs to map public <-> tunnel IPs

- Dynamic multicast: enables routing protocols (EIGRP/OSPF)

# ⭐ STEP 2 — SPOKE1 CONFIGURATION (R2)

R2(config)# interface tunnel0

R2(config-if)# ip address 172.16.0.2 255.255.255.0

R2(config-if)# no ip redirects

R2(config-if)# tunnel source f0/0

R2(config-if)# tunnel mode gre multipoint

R2(config-if)# ip nhrp network-id 10

R2(config-if)# ip nhrp map 172.16.0.1 10.1.1.1

R2(config-if)# ip nhrp map multicast 10.1.1.1

R2(config-if)# ip nhrp nhs 172.16.0.1

R2(config-if)# exit

Purpose:

- Map hub public IP to tunnel IP

- Set hub as NHRP server (NHS)

# ⭐ STEP 3 — SPOKE2 CONFIGURATION (R3)

R3(config)# interface tunnel0

R3(config-if)# ip address 172.16.0.3 255.255.255.0

R3(config-if)# no ip redirects

R3(config-if)# tunnel source f0/0

R3(config-if)# tunnel mode gre multipoint

R3(config-if)# ip nhrp network-id 10

R3(config-if)# ip nhrp map 172.16.0.1 10.1.1.1

R3(config-if)# ip nhrp map multicast 10.1.1.1

R3(config-if)# ip nhrp nhs 172.16.0.1

R3(config-if)# exit

# ⭐ STEP 4 — IPsec CONFIGURATION (SHARED ON ALL ROUTERS)

### 🔵 ISAKMP (IKE Phase-1)

crypto isakmp policy 10

encryption aes

hash sha

authentication pre-share

group 2

exit

### 🔵 Pre-shared key

(Do this on all routers)

crypto isakmp key ciscoR3 address 0.0.0.0 0.0.0.0

➡️ Accepts any DMVPN peer.

# ⭐ STEP 5 — IPsec Transform Set (Phase-2)

crypto ipsec transform-set DMVPN-SET esp-aes esp-sha-hmac

mode transport

exit

➡️ DMVPN uses transport mode (not tunnel).

# ⭐ STEP 6 — DMVPN PROFILE

crypto ipsec profile DMVPN-PROFILE

set transform-set DMVPN-SET

exit

# ⭐ STEP 7 — APPLY IPsec TO DMVPN TUNNEL

### On R1, R2, R3:

interface tunnel0

tunnel protection ipsec profile DMVPN-PROFILE

exit

➡️ Encrypts GRE traffic with IPsec.

# ⭐ STEP 8 — VERIFICATION

### NHRP Status:

show ip nhrp

### DMVPN Status:

show dmvpn

### IPsec SAs:

show crypto isakmp sa

show crypto ipsec sa

# ⭐ ONE-PAGE COMPLETE EXAM SHEET — DMVPN + IPsec

###############################

# HUB (R1)

###############################

int tunnel0

ip address 172.16.0.1 255.255.255.0

no ip redirects

tunnel source f1/1

tunnel mode gre multipoint

ip nhrp network-id 10

ip nhrp map multicast dynamic

tunnel protection ipsec profile DMVPN-PROFILE

exit

###############################

# SPOKE1 (R2)

###############################

int tunnel0

ip address 172.16.0.2 255.255.255.0

no ip redirects

tunnel source f0/0

tunnel mode gre multipoint

ip nhrp network-id 10

ip nhrp map 172.16.0.1 10.1.1.1

ip nhrp map multicast 10.1.1.1

ip nhrp nhs 172.16.0.1

tunnel protection ipsec profile DMVPN-PROFILE

exit

###############################

# SPOKE2 (R3)

###############################

int tunnel0

ip address 172.16.0.3 255.255.255.0

no ip redirects

tunnel source f0/0

tunnel mode gre multipoint

ip nhrp network-id 10

ip nhrp map 172.16.0.1 10.1.1.1

ip nhrp map multicast 10.1.1.1

ip nhrp nhs 172.16.0.1

tunnel protection ipsec profile DMVPN-PROFILE

exit

###############################

# IPSEC FOR ALL ROUTERS

###############################

crypto isakmp policy 10

encryption aes

hash sha

authentication pre-share

group 2

exit

crypto isakmp key ciscoR3 address 0.0.0.0 0.0.0.0

crypto ipsec transform-set DMVPN-SET esp-aes esp-sha-hmac

mode transport

exit

crypto ipsec profile DMVPN-PROFILE

set transform-set DMVPN-SET

exit

###############################

# VERIFICATION

###############################

show ip nhrp

show dmvpn

show crypto isakmp sa

show crypto ipsec sa

# TOPIC 10 — Configuring IPv6 Security

# 🖼 TOPOLOGY (Based on Your Diagram)

R1 ---------------- R2 ---------------- R3

(EIGRPv6)         (Redistribution)        (OSPFv3)

R1–R2 link: 2001:DB8:12::/64

R2–R3 link: 2001:DB8:23::/64

R1 interface: 2001:DB8:12::1

R2 interface: 2001:DB8:12::2 and 2001:DB8:23::1

R3 interface: 2001:DB8:23::2

- R1–R2 uses EIGRPv6

- R2–R3 uses OSPFv3 (Area 0)

- R2 redistributes both ways

# ⭐ R1 CONFIGURATION

## 🔵 Enable IPv6 routing

R1(config)# ipv6 unicast-routing

➡️ Enables IPv6 forwarding.

## 🔵 Configure interface IPv6

R1(config)# interface serial1/0

R1(config-if)# ipv6 address 2001:DB8:12::1/64

R1(config-if)# no shutdown

➡️ Assign IPv6 to R1–R2 link.

## 🔵 Enable EIGRPv6 on R1

R1(config)# ipv6 router eigrp 100

R1(config-rtr)# eigrp router-id 1.1.1.1

R1(config-rtr)# no shutdown

R1(config-rtr)# exit

➡️ Creates EIGRPv6 process.

R1(config)# interface serial1/0

R1(config-if)# ipv6 eigrp 100

➡️ Enables EIGRP on interface.

# ⭐ R2 CONFIGURATION

## 🔵 Enable IPv6 routing

R2(config)# ipv6 unicast-routing

## 🔵 R2 Interfaces

### R1–R2 link:

R2(config)# interface serial1/0

R2(config-if)# ipv6 address 2001:DB8:12::2/64

R2(config-if)# no shutdown

R2(config-if)# ipv6 eigrp 100

### R2–R3 link:

R2(config)# interface serial1/1

R2(config-if)# ipv6 address 2001:DB8:23::1/64

R2(config-if)# no shutdown

R2(config-if)# ipv6 ospf 10 area 0

## 🔵 R2 EIGRPv6 Router Process

R2(config)# ipv6 router eigrp 100

R2(config-rtr)# eigrp router-id 2.2.2.2

R2(config-rtr)# no shutdown

R2(config-rtr)# exit

## 🔵 R2 OSPFv3 Router Process

R2(config)# ipv6 router ospf 10

R2(config-rtr)# router-id 2.2.2.2

R2(config-rtr)# exit

# ⭐ R3 CONFIGURATION

## 🔵 Enable IPv6 routing

R3(config)# ipv6 unicast-routing

## 🔵 Configure interface + OSPFv3

R3(config)# interface serial1/1

R3(config-if)# ipv6 address 2001:DB8:23::2/64

R3(config-if)# no shutdown

R3(config-if)# ipv6 ospf 10 area 0

## 🔵 OSPFv3 Router Process

R3(config)# ipv6 router ospf 10

R3(config-rtr)# router-id 3.3.3.3

R3(config-rtr)# exit

# ⭐ REDISTRIBUTION (Done on R2)

## 🔵 Redistribute OSPFv3 → EIGRPv6

R2(config)# ipv6 router eigrp 100

R2(config-rtr)# redistribute ospf 10 metric 10000 100 255 1 1500

R2(config-rtr)# exit

➡️ Inject OSPFv3 routes into EIGRPv6.

## 🔵 Redistribute EIGRPv6 → OSPFv3

R2(config)# ipv6 router ospf 10

R2(config-rtr)# redistribute eigrp 100 metric-type 1

R2(config-rtr)# exit

➡️ Inject EIGRPv6 routes into OSPFv3.

# ⭐ VERIFICATION

show ipv6 eigrp neighbor

show ipv6 ospf neighbor

show ipv6 route

# ⭐ ONE-PAGE FINAL EXAM SHEET — TOPIC 10

###########################################

# R1

###########################################

ipv6 unicast-routing

int s1/0

ipv6 address 2001:DB8:12::1/64

no shutdown

ipv6 eigrp 100

exit

ipv6 router eigrp 100

eigrp router-id 1.1.1.1

no shutdown

exit

###########################################

# R2

###########################################

ipv6 unicast-routing

int s1/0

ipv6 address 2001:DB8:12::2/64

no shutdown

ipv6 eigrp 100

exit

int s1/1

ipv6 address 2001:DB8:23::1/64

no shutdown

ipv6 ospf 10 area 0

exit

ipv6 router eigrp 100

eigrp router-id 2.2.2.2

no shutdown

redistribute ospf 10 metric 10000 100 255 1 1500

exit

ipv6 router ospf 10

router-id 2.2.2.2

redistribute eigrp 100 metric-type 1

exit

###########################################

# R3

###########################################

ipv6 unicast-routing

int s1/1

ipv6 address 2001:DB8:23::2/64

no shutdown

ipv6 ospf 10 area 0

exit

ipv6 router ospf 10

router-id 3.3.3.3

exit

###########################################

# VERIFICATION

###########################################

show ipv6 eigrp neighbor

show ipv6 ospf neighbor

show ipv6 route

# TOPIC 11 — CONFIGURING INTER-VLAN ROUTING (ROUTER-ON-A-STICK)

# 🖼 TOPOLOGY (From your notes)

Router (R1)

|

Gi0/0 (TRUNK)

|

---------

| Switch |

---------

VLAN10 (HR)        VLAN20 (ACCOUNTS)

192.168.10.0/24      192.168.20.0/24

PCs:

HR-PC → 192.168.10.x

AC-PC → 192.168.20.x

# ⭐ STEP 1 — SWITCH CONFIGURATION (VLANs & Access Ports)

## 🔵 Create VLAN 10 (HR)

Switch(config)# vlan 10

Switch(config-vlan)# name HR

Switch(config-vlan)# exit

➡️ Creates HR VLAN.

## 🔵 Create VLAN 20 (ACCOUNT)

Switch(config)# vlan 20

Switch(config-vlan)# name ACCOUNT

Switch(config-vlan)# exit

➡️ Creates Account VLAN.

## 🔵 Assign Access Ports for VLAN 10

(Your diagram: ports F0/2–3)

Switch(config)# interface range fastEthernet 0/2 - 3

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 10

Switch(config-if)# exit

➡️ Connect HR PCs to VLAN 10.

## 🔵 Assign Access Ports for VLAN 20

(Your diagram: ports F0/4–5)

Switch(config)# interface range fastEthernet 0/4 - 5

Switch(config-if)# switchport mode access

Switch(config-if)# switchport access vlan 20

Switch(config-if)# exit

➡️ Connect Accounts PCs to VLAN 20.

# ⭐ STEP 2 — MAKE ROUTER INTERFACE A TRUNK (Router-On-A-Stick)

## 🔵 On Router R1

### Make F0/1 (or Gi0/0) a trunk

R1(config)# interface fastEthernet 0/1

R1(config-if)# switchport mode trunk

R1(config-if)# exit

➡️ Allows VLAN tagging between switch and router.

# ⭐ STEP 3 — CREATE ROUTER SUBINTERFACES

Router uses subinterfaces to route between VLANs.

## 🔵 SUBINTERFACE FOR VLAN 10 (HR)

R1(config)# interface gigabitEthernet 0/0.10

R1(config-subif)# encapsulation dot1Q 10

R1(config-subif)# ip address 192.168.10.1 255.255.255.0

R1(config-subif)# no shutdown

R1(config-subif)# exit

➡️ Default gateway for all HR-PCs.

## 🔵 SUBINTERFACE FOR VLAN 20 (ACCOUNT)

R1(config)# interface gigabitEthernet 0/0.20

R1(config-subif)# encapsulation dot1Q 20

R1(config-subif)# ip address 192.168.20.1 255.255.255.0

R1(config-subif)# no shutdown

R1(config-subif)# exit

➡️ Default gateway for Accounts PCs.

# ⭐ NOW INTER-VLAN ROUTING WORKS

PCs in VLAN 10 can reach VLAN 20 through router.

# ⭐ ONE-PAGE FINAL EXAM SHEET — TOPIC 11

####################################

# SWITCH

####################################

vlan 10

name HR

exit

vlan 20

name ACCOUNT

exit

interface range fa0/2 - 3

switchport mode access

switchport access vlan 10

exit

interface range fa0/4 - 5

switchport mode access

switchport access vlan 20

exit

####################################

# ROUTER (Router-on-a-Stick)

####################################

interface fa0/1

switchport mode trunk

exit

interface gi0/0.10

encapsulation dot1Q 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

interface gi0/0.20

encapsulation dot1Q 20

ip address 192.168.20.1 255.255.255.0

no shutdown

exit

# TOPIC 12 — Port Security, EtherChannel, DHCP Snooping & BPDU Guard

# 🖼 TOPOLOGY (Based on your diagram)

Switch1 <—— EtherChannel ——> Switch2

PC1 connected to Switch1 F0/1

PC2 connected to Switch2 F0/1

Also includes:

• Port Security on Access Port

• DHCP Snooping

• BPDU Guard

# ⭐ PART 1 — ETHERCHANNEL CONFIGURATION (LACP / ACTIVE MODE)

### 🎯 Goal: Combine multiple ports (like F0/1 & F0/2) into a single logical link (Port-Channel 1)

## 🔵 On Both Switches (SW1 & SW2)

### Configure Physical Interfaces (F0/1 & F0/2)

interface range fastEthernet 0/1 - 2

switchport mode trunk

channel-group 1 mode active

exit

✔ Purpose:

- mode trunk → carry multiple VLANs

- channel-group 1 mode active → enables LACP EtherChannel

### Configure the Logical Port-Channel

interface port-channel 1

switchport mode trunk

exit

✔ Purpose: Makes Port-Channel behave as trunk interface.

# ⭐ PART 2 — PORT SECURITY CONFIGURATION (Access Port)

### 🎯 Goal: Allow only one MAC address, disable port on violation

### Example on SW1 F0/1

interface fastEthernet 0/1

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security maximum 1

switchport port-security violation shutdown

exit

✔ Purpose Line-by-Line:

- mode access → required for port security

- port-security → enables security

- sticky → learns MAC dynamically and saves it

- maximum 1 → only one MAC allowed

- violation shutdown → port goes off if another MAC detected

# ⭐ PART 3 — DHCP SNOOPING CONFIGURATION

### 🎯 Goal: Stop rogue DHCP servers and protect clients

## On Switch (Example SW1)

### Enable Globally

ip dhcp snooping

✔ Protects LAN from fake DHCP servers.

### Enable Snooping on VLANs

ip dhcp snooping vlan 10,20

✔ Apply DHCP security for HR & Accounts VLAN.

### Trust Uplink Ports

interface port-channel1

ip dhcp snooping trust

exit

✔ Trust only uplinks, NOT access ports.

### Block speed-too-fast DHCP replies

ip dhcp snooping limit rate 10

✔ Prevents DHCP starvation attacks.

# ⭐ PART 4 — BPDU GUARD CONFIGURATION

### 🎯 Goal: Protect switch from unauthorized switches on access ports

### On Access Ports Only (ex: F0/1)

interface fastEthernet 0/1

spanning-tree bpduguard enable

exit

✔ If BPDU detected → port shuts down (prevents STP attack).

# ⭐ FULL ONE-PAGE EXAM SHEET — TOPIC 12

#########################

# ETHERCHANNEL (LACP)

#########################

interface range f0/1 - 2

switchport mode trunk

channel-group 1 mode active

exit

interface port-channel 1

switchport mode trunk

exit

#########################

# PORT SECURITY

#########################

interface f0/1

switchport mode access

switchport port-security

switchport port-security mac-address sticky

switchport port-security maximum 1

switchport port-security violation shutdown

exit

#########################

# DHCP SNOOPING

#########################

ip dhcp snooping

ip dhcp snooping vlan 10,20

interface port-channel 1

ip dhcp snooping trust

exit

OR

interface g0/1

ip dhcp snooping trust

exit

ip dhcp snooping limit rate 10

#########################

# BPDU GUARD

#########################

interface f0/1

spanning-tree bpduguard enable

exit